# Atlas-2 Pro — Installation Guide

## What's in the Box

Each Atlas-2 Pro shipment contains:

The Atlas-2 Pro robot (pre-assembled).

A docking station with a 2-meter detachable power cable.

A wireless charging mat for the on-board tablet.

Three RFID elevator beacons.

A printed quick-start card.

A USB-C cable for diagnostic access.

## Site Requirements

The Atlas-2 Pro requires:

A flat operating surface with a maximum slope of 5°.

Wi-Fi 6 coverage with at least -65 dBm signal strength in all

intended operating zones.

A standard EU-style 230 V / 50 Hz power outlet within 1.5 meters of

the docking station.

Door frames at least 84 cm wide.

For multi-floor operation, an elevator with an open API or a

Wi-Fi-controlled relay (we provide a recommended hardware list on

request).

## First Boot

Place the docking station against a wall in your designated charging

zone. Plug it in. The status LED turns solid green when ready.

Lift the Atlas-2 Pro onto the dock. It will boot automatically; the

first boot takes approximately 4 minutes.

The on-board tablet displays a six-digit pairing code. Enter this

code at https://manage.acmerobotics.example to associate the robot

with your account.

Run the floor mapping wizard. Mapping a typical hotel floor takes

12 to 25 minutes depending on layout complexity.

## Common Issues

**Robot does not charge on the dock**: ensure the contact pads on the

underside are clean. Wipe them with isopropyl alcohol if needed.

**Wi-Fi disconnects on Floor 4**: most often a coverage problem.

Walk the route with the diagnostic app in monitor mode; signal must

stay above -70 dBm.

**Elevator integration fails**: confirm the elevator beacon is

flashing blue. A solid red light indicates a flat coin-cell battery

(CR2032).

## Firmware Updates

Firmware updates are pushed automatically every Tuesday at 03:00 local

time. To install a critical security patch immediately, run

atlas-update --force from the maintenance console. Updates take

under 6 minutes and the robot is unavailable during the update.